# TracPhone® V30
Preventative Maintenance Check (PMC)

| Vessel Name |  |
| --- | --- |
| Customer/Fleet Management Company |  |
| PMC Case Number |  |
| Service Company |  |
| Service Technician |  |
| Service Technician Phone No. |  |
| Service Technician Email Address |  |
| Date of PMC |  |

Table of Contents

Instructions	2

1	Initial System Performance	3

2	Hardware Conditions	5

3	Software Upgrade and Test	19

4	Cleanliness Inspection	20

5	Customer Approval and Submittal to KVH	20

TracPhone V30 Preventative Maintenance Check (PMC) Report	21

| If you need assistance completing this checklist, or if an issue arises during the PMC, please contact KVH Technical Support: |  |
| --- | --- |
| Within Continental U.S.A.: 1 866 701-7103 Worldwide: +1 401 851-3806 Email: mvbsupport@kvh.com |  |

# Instructions

Follow this checklist to complete the preventative maintenance check (PMC) for a TracPhone V30 antenna system. Conduct each check and enter all requested information, where applicable. When entering measurements, don’t forget to include the associated units (e.g., feet or meters). When you are done, please email the completed checklist, signed report, equipment photos, and antenna data file to KVH at PMC@kvh.com.

You will need the following items to complete the PMC:

| - Measuring tape - Digital camera (or mobile device with camera) - #1 and #2 Phillips-head screwdrivers - Utility knife - Scissors - 7/16" open-end torque wrench set to 15 in-lbs - 1/2" socket and 7/16" socket - 4mm hex key - Driver with T10 and T20 Torx bits - Adjustable torque driver | - Tool kits for terminating RG-58 and LMR-240 cables with TNC and N connectors - Voltmeter - Duct tape - Shop towels - Laptop PC with the latest V30 software update file downloaded from the KVH Partner Portal - Ethernet cable - PMC hardware kit (KVH part no. 72-0952) |
| --- | --- |

For each item in the PMC checklist, check the box in the appropriate column:

| Column | Definition |
| --- | --- |
|  | The item was checked and verified to be without any issues. |
|  | The item was checked and has a potential issue. Enter an explanation in the Notes section at the end of the table. Be sure to reference the sequence number. |
| N/A | The item could not be checked due to an unforeseen circumstance. Enter an explanation in the Notes section at the end of the table. Be sure to reference the sequence number. |

# Initial System Performance

| No. | Check |  |  | N/A |
| --- | --- | --- | --- | --- |
|  | Apply power to the VSAT-Hub and verify that the system initializes, finds and tracks the satellite, then logs into the mini-VSAT Broadband service. Verify that the three LEDs are lit green. If the antenna cannot track the satellite due to blockage, verify that all three LEDs are lit or flashing green. | ☐ | ☐ | ☐ |
|  | WARNING! Use caution when measuring voltages. Always comply with electrical safety best practices and the meter manufacturer’s operating instructions. Failure to do so risks electric shock, potentially resulting in DEATH. Ask the ship’s electrician for assistance, if necessary. |  |  |  |
|  | Measure the AC voltage between the VSAT-Hub’s chassis ground (any metal surface) and ship’s ground. Verify that it measures less than 2 VAC. A higher voltage indicates a dangerous floating ground condition. | ☐ | ☐ | ☐ |
|  | AC voltage between chassis ground and ship’s ground: |  |  |  |
|  | Measure the DC voltage between the VSAT-Hub’s chassis ground (any metal surface) and ship’s ground. Verify that it measures less than 2 VDC. A higher voltage indicates a dangerous floating ground condition. | ☐ | ☐ | ☐ |
|  | DC voltage between chassis ground and ship’s ground: |  |  |  |
|  | WARNING! If you measure 2 volts or greater between the chassis ground and ship’s ground, notify the ship’s electrician or authorized vessel representative immediately. This is a dangerous condition. Do not touch the VSAT-Hub rear panel or connect anything to it until the problem is fixed. |  |  |  |
|  | Connect your laptop to the system, open a web browser, and verify that it can connect to the Internet. If you are unable to connect to the vessel’s network, try connecting directly to one of the Ethernet ports on the back of the VSAT-Hub. If the antenna cannot track the satellite due to blockage, select N/A. | ☐ | ☐ | ☐ |
|  | Locate a phone that is connected to the VSAT-Hub and verify that there is a dial tone. If the antenna cannot track the satellite due to blockage, select N/A. | ☐ | ☐ | ☐ |
|  | Access the web interface (https://kvhonboard.com or 192.168.5.1) and verify that the system is not reporting any errors. | ☐ | ☐ | ☐ |
|  | Record a new data file of the antenna’s startup and self-test routine (capture 10 minutes of data) following the instructions below. Save this file under the name “PMC_V30_<vessel>,” where <vessel> is the first 6 characters of the vessel’s name. Submit this data file to KVH upon completion of the PMC. At the Support page of the web interface, restart the system (Restart All). Wait at least 10 minutes. Then download the Operational Log from the System Logs menu on the Support page. | ☐ | ☐ | ☐ |
|  | Turn off the system and unplug the VSAT-Hub’s power cable to disconnect all power from the antenna. Follow all shipboard lockout-tagout procedures, where applicable. | ☐ | ☐ | ☐ |
|  | WARNING! To prevent injury, be sure to disconnect power from the system before proceeding and keep it disconnected for the duration of this procedure. Failure to disconnect power risks electric shock, potentially resulting in DEATH. |  |  |  |
| Notes: |  |  |  |  |

# Hardware Conditions

| No. | Check |  |  |  | N/A |
| --- | --- | --- | --- | --- | --- |
| Antenna Checks |  |  |  |  |  |
|  | Verify that the antenna is mounted securely and the mounting surface appears strong enough to support the antenna’s weight and rigid enough to prevent antenna vibration and withstand cumulative forces related to expected operating conditions, such as ice, snow, wind, etc. |  | ☐ | ☐ | ☐ |
|  | Take a photo of the antenna’s mounting area and submit it to KVH upon completion of the PMC. |  | ☐ | ☐ | ☐ |
|  | Inspect the baseplate mounting feet. Make sure they haven’t deteriorated. |  | ☐ | ☐ | ☐ |
|  | Verify that the antenna is mounted outside the radar’s beam path. If the antenna does not clearly meet these requirements, measure and record the distances listed below and note the radar type/power level. |  | ☐ | ☐ | ☐ |
|  | Distance A: |  |  |  |  |
|  | Distance B: |  |  |  |  |
|  | Radar Type: |  |  |  |  |
|  | Radar Power: |  |  |  |  |
|  | If possible, inspect the above deck cable run to the antenna. Check the antenna cable for damage, kinks, or excessive bends. Also verify that there is a sufficient service loop and strain relief. In the space below, enter the type of coax cable used, and specify whether it is rated for 50 ohms or 75 ohms. If you are unable to identify the cable rating, or if a 75-ohm cable is used, add a note in the Notes section at the end of this table. |  | ☐ | ☐ | ☐ |
|  | Cable type/rating: |  |  |  |  |
|  | If a pigtail cable is connected between the antenna cable (from VSAT-Hub) and the antenna, verify that its connection is properly weather-sealed. If the pigtail is not properly weather-sealed, carefully remove the seal and check the connector for potential water ingress. If necessary, replace the connector, reapply silicone grease, reconnect the cable (tighten to proper torque), and reseal the connection using self-vulcanizing tape. Be sure to use the proper stripping/crimping tools and follow the manufacturer’s instructions to terminate the cable. |  | ☐ | ☐ | ☐ |
|  | Remove the radome. If the plastic screw caps are painted, set them aside for future use. Inspect the radome. Look for cracks or other damage. If cracks are found, seal them temporarily with tape, such as duct tape. Also make sure the radome’s gasket is intact. |  | ☐ | ☐ | ☐ |
|  | Inspect the baseplate. Look for cracks or other damage. If cracks are found, seal them temporarily with tape, such as duct tape. |  | ☐ | ☐ | ☐ |
| Checks 2.9 through 2.11 require you to detach the antenna from the mounting surface to inspect items on the antenna’s base. If the antenna cannot be detached easily within the PMC time constraints, you may select N/A for these items and skip to check no. 2.12. |  |  |  |  |  |
|  | If possible, detach the antenna from its mounting surface (save the hardware) and verify that the cable connection on the bottom of the antenna is properly weather-sealed. If the weather seal is insufficient, carefully remove the seal and check the connector for potential water ingress. If necessary, replace the connector, reapply silicone grease, reconnect the cable (hand-tighten), and reseal the connection with self-vulcanizing tape or silicone sealant. Be sure to use the proper stripping/crimping tools and follow the manufacturer’s instructions to terminate the cable. If the baseplate connector is damaged, contact KVH Technical Support. |  | ☐ | ☐ | ☐ |
|  | Inspect the foam seal on the mounting surface. Verify that the foam is not degraded. Replace it if necessary. |  | ☐ | ☐ | ☐ |
|  | Secure the antenna to the mounting surface using the hardware you removed earlier. Be sure to first apply anti-seize compound to the threads of the bolts. Tighten to between 25-30 ft-lbs (34-41 N-m) of torque. |  | ☐ | ☐ | ☐ |
|  | Inspect all visible cables inside the antenna. Look for damage or loose connections. Tighten any loose connections. |  | ☐ | ☐ | ☐ |
|  | Inspect all visible internal antenna hardware. Look for loose or missing fasteners. Tighten any loose fasteners. |  | ☐ | ☐ | ☐ |
|  | Inspect the internal antenna assembly for any signs of corrosion or water damage. If corrosion or water damage exists, identify the affected components below and take photos. Submit the photos to KVH upon completion of the PMC. |  | ☐ | ☐ | ☐ |
|  | Corroded components: |  |  |  |  |
|  | Water-damaged components: |  |  |  |  |
|  | Locate the antenna’s serial number label on the frame near the azimuth (AZ) motor and enter the 9-digit number below. |  | ☐ | ☐ | ☐ |
|  | Antenna serial number: |  |  |  |  |
|  | Inspect the cable connections at the VSAT modem. Adjust the position of the RF cable connectors, if necessary, as shown below. Then tighten them to 15 in-lbs of torque. |  | ☐ | ☐ | ☐ |
|  | Inspect the BUC. Verify that its cable connector is not loose. Tighten if necessary. |  | ☐ | ☐ | ☐ |
|  | Inspect the GPS. Verify that the GPS is secured to the reflector and its cable is free of damage or kinks. |  | ☐ | ☐ | ☐ |
|  | At the front of the reflector, inspect the feed tube. Verify that it is secured properly. Tighten any loose fasteners. |  | ☐ | ☐ | ☐ |
|  | Verify that the internal LMR-240 cable is securely connected to the main board. Hand-tighten if necessary. |  | ☐ | ☐ | ☐ |
|  | While slowly rotating the antenna assembly by hand for one complete revolution (360°), inspect the azimuth belt as best you can through the cutouts in the rotating plate. Look for tears or missing teeth. Verify that the belt is seated properly around the pulleys. Also verify that the antenna assembly moves smoothly without any obstructions. |  | ☐ | ☐ | ☐ |
|  | Inspect the azimuth motor. Verify that it is mounted securely and its cable connector is not loose. Tighten if necessary. |  | ☐ | ☐ | ☐ |
|  | Inspect the elevation (EL) belt. Look for tears or missing teeth. Verify that the belt is seated properly around the pulleys. |  | ☐ | ☐ | ☐ |
|  | Slowly tilt the reflector downward until the elevation bracket engages the elevation limit switch. Verify that the limit switch closes properly by listening for the click as the bracket presses the actuator against the switch. |  | ☐ | ☐ | ☐ |
|  | Slowly tilt the reflector to the upper end of its range of motion. Verify that the elevation mechanism moves freely with no obstructions. |  | ☐ | ☐ | ☐ |
|  | Check the tension of the elevation belt at three antenna elevations: 7.7° (lowest elevation), 45° (mid-range), and 75° (highest elevation). Verify that the tension is consistent and the belt is tight enough to stay completely engaged with the pulleys. |  | ☐ | ☐ | ☐ |
|  | Inspect the elevation motor. Verify that it is mounted securely and its cable connector is not loose. Tighten if necessary. |  | ☐ | ☐ | ☐ |
|  | Inspect the skew belt. Look for tears or missing teeth. Verify that the belt is seated properly around the pulleys. |  | ☐ | ☐ | ☐ |
|  | Inspect the skew motor. Verify that it is mounted securely and its cable connector is not loose. Tighten if necessary. |  | ☐ | ☐ | ☐ |
|  | Slowly rotate the RF feed assembly clockwise until the skew limit switch is engaged. Verify that the limit switch closes properly by listening for the click as the actuator presses against the switch. |  | ☐ | ☐ | ☐ |
|  | Starting from the point at which the skew limit switch is engaged, slowly rotate the RF feed assembly 180° counter-clockwise. Verify that the RF feed assembly moves freely with no obstructions. Starting position (limit switch engaged) Ending position (180° counter-clockwise) |  | ☐ | ☐ | ☐ |
|  | Check the skew belt tension at three skew positions: - Rotated fully clockwise (point at which the skew limit switch is engaged) - Rotated 90° counter-clockwise - Rotated 180° counter-clockwise Verify that the tension is consistent and the belt is tight enough to stay completely engaged with the pulleys. |  | ☐ | ☐ | ☐ |
|  | Inspect the skew cable wrap. Look for tears, fraying, and/or wire breaks. |  | ☐ | ☐ | ☐ |
|  | Inspect the LNB. Verify that it is mounted securely and its cable connector is not loose. Tighten if necessary. |  | ☐ | ☐ | ☐ |
|  | Inspect the fans mounted to the main board and VSAT modem. Wipe away any debris from the fan blades. |  | ☐ | ☐ | ☐ |
|  | Make sure you have not left any tools or debris inside the antenna. Then reattach the radome to the baseplate using new hardware and screw caps. If painted screw caps were used, reinstall them instead of the supplied caps. |  | ☐ | ☐ | ☐ |

| No. | Check |  |  | N/A |
| --- | --- | --- | --- | --- |
| VSAT-Hub Checks |  |  |  |  |
|  | WARNING! Use caution when measuring voltages. Always comply with electrical safety best practices and the meter manufacturer’s operating instructions. Failure to do so risks electric shock, potentially resulting in DEATH. Ask the ship’s electrician for assistance, if necessary. |  |  |  |

|  | Reapply DC power and measure the DC voltage input to the VSAT-Hub. Verify that 10-30 VDC is being supplied to the VSAT-Hub. | ☐ | ☐ | ☐ |
| --- | --- | --- | --- | --- |
|  | DC voltage at VSAT-Hub power input: |  |  |  |

|  | Make sure a ground wire is connected to the ground screw on the VSAT-Hub’s rear panel. | ☐ | ☐ | ☐ |
| --- | --- | --- | --- | --- |
| Notes: |  |  |  |  |

# Software Upgrade and Test

| No. | Requirement |  |  | N/A |
| --- | --- | --- | --- | --- |
|  | Reconnect the power cable to the VSAT-Hub. Turn it on and verify that the system initializes, finds and tracks the satellite, then logs into the mini-VSAT Broadband service. Verify that the three LEDs are lit green. If the antenna cannot track the satellite due to blockage, verify that all three LEDs are lit or flashing green. | ☐ | ☐ | ☐ |
|  | Note the serial numbers for the VSAT-Hub and VSAT modem. You can find these serial numbers on the Support page of the system’s web interface. Record the numbers below. | ☐ | ☐ | ☐ |
|  | VSAT-Hub serial number: |  |  |  |
|  | VSAT modem serial number: |  |  |  |
|  | Verify that the system is receiving heading data from an external NMEA 0183 or NMEA 2000 source. On the Home page of the system’s web interface, look for a true heading value in the Real-time Vessel & VSAT Satellite Display section. | ☐ | ☐ | ☐ |
|  | Update the system software to the latest revision. Refer to the instructions in the system’s Help (see Maintenance > Software Updates). | ☐ | ☐ | ☐ |
|  | Perform this step with the authorized vessel representative. Using your laptop connected to the system, open a web browser, and verify that it can connect to the Internet. If the antenna cannot track the satellite due to blockage, select N/A. | ☐ | ☐ | ☐ |
|  | Perform this step with the authorized vessel representative. Locate a phone that is connected to the VSAT-Hub and verify that there is a dial tone. If the antenna cannot track the satellite due to blockage, select N/A. | ☐ | ☐ | ☐ |
|  | Disconnect your laptop from the VSAT-Hub and reconnect any cables that you disconnected earlier. Inspect the cable routing behind the VSAT-Hub and ensure all cables are routed neatly, securely, and without any kinks or excessive bends. | ☐ | ☐ | ☐ |
|  | Take photos of the VSAT-Hub’s wiring and mounting area and submit them to KVH upon completion of the PMC. | ☐ | ☐ | ☐ |
| Notes: |  |  |  |  |

# Cleanliness Inspection

Before you depart the vessel, make sure you leave nothing behind. Make sure all packaging materials have been disposed of properly, all tools have been packed up, and all work areas have been cleaned up.

# Customer Approval and Submittal to KVH

Fill out and sign the report on the next page and ask the customer to sign it for approval. Then scan the signed report and email it to KVH, along with this entire completed PMC checklist, the antenna data file, and equipment installation photos to PMC@kvh.com.

# TracPhone V30 Preventative Maintenance Check (PMC) Report

| Vessel Name: |  | Date: |  |
| --- | --- | --- | --- |

## Summary of Findings

Your PMC service encompassed over 50 items. The checklist below summarizes the results.

| Tasks Completed: |  |  | N/A |
| --- | --- | --- | --- |
| Measured the input voltage and verified the system is properly grounded | ☐ | ☐ | ☐ |
| Verified antenna is securely and safely mounted and meets all installation best practices | ☐ | ☐ | ☐ |
| Checked cables, both inside and outside the dome, for loose connections, corroded connectors, kinks, or other damage | ☐ | ☐ | ☐ |
| Inspected the internal antenna mechanism inside the dome, looking for any signs of wear and tear or potential points of failure | ☐ | ☐ | ☐ |
| Checked all motors, belts, and limit switches and verified antenna has full range of motion in azimuth, elevation, and skew | ☐ | ☐ | ☐ |
| Updated the software to the latest version | ☐ | ☐ | ☐ |
| Tested the system for proper operation and captured an antenna data file | ☐ | ☐ | ☐ |
| Issues Found: |  |  |  |

To schedule a follow-up service to address any issues, contact KVH Technical Support (mvbsupport@kvh.com).

| Service Technician I attest that I completed this PMC in accordance with KVH requirements. | Signature: |
| --- | --- |
|  | Print name: |
| Authorized Vessel Representative I affirm that the PMC has been completed to my satisfaction. | Signature: |
|  | Print name: |